200 staff

- Production 400

  - (100 staff  + BYOD + expansion)

  - Very likely to grow. Largest growth expected

- Warehouse 50

  - (20 staff + BYOD + expansion)

  - Not expected to grow too much, very stable department

- Sales & marketing 60

  - (20staff + BYOD + expansion)

  - Medium growth, BYOD heavy

- Finance 30

  - (10 staff + BYOD + expansion)

  - Not expected to drastically grow, small & stable

- R&D 100

  - (30 staff + BYOD + growth)

  - Moderate expansion on a new product

- Management 80

  - (30 staff + BYOD + expansion)

  - Never need more management :) – limited growth

- MANAGEMENT_VLAN 200

  - Arbitrarily allocated 200 hosts, definite future expansion buffer

  - Deals with sensitive traffic, SSH/Telnet etc

- Guest/BYOD 200

  - Even if all departments BYOD, external office users (from outside) still may wish to temporarily connect

Dedicated computer connections (laptops & PCs). Certain staff (sales & marketing) need BYOD for future expansion

Introduction

Limitations & Assumptions:

Network Topology & Design

IP Addressing & VLAN Configuration

| Department & VLAN ID | Hosts needed | Hosts available | Unused Hosts | Network ID | Useable Hosts | Broadcast ID | Mask |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production VLAN 10 | 400 | 510 | 110 | 10.0.0.0 | 10.0.0.1 – 10.0.1.254 | 10.0.1.255 | /23 255.255.254.0 |
| Guest/BYOD VLAN 20 | 200 | 254 | 54 | 10.0.2.0 | 10.0.2.1 - 10.0.2.254 | 10.0.2.255 | /24 |
| MANAGEMENT_VLAN VLAN 30 | 200 | 254 | 54 | 10.0.3.0 | 10.0.3.1 - 10.0.3.254 | 10.0.3.255 | /24 |
| R&D VLAN 40 | 100 | 126 | 26 | 10.0.4.0 | 10.0.4.1 - 10.0.4.126 | 10.0.4.127 | /25 |
| Management (executives) VLAN 50 | 80 | 126 | 46 | 10.0.4.128 | 10.0.4.129 - 10.0.4.254 | 10.0.4.255 | /25 |
| Sales & Marketing VLAN 60 | 60 | 62 | 2 | 10.0.5.0 | 10.0.5.1 - 10.0.5.62 | 10.0.5.63 | /26 |
| Warehouse VLAN 70 | 50 | 62 | 12 | 10.0.5.64 | 10.0.5.65 - 10.0.5.126 | 10.0.5.127 | /26 |
| Finance VLAN 80 | 30 | 30 | 0 | 10.0.5.128 | 10.0.5.129 - 10.0.5.158 | 10.0.5.159 | /27 |
| Native VLAN 99 | No Addressing Required |  |  |  |  |  |  |

Wireless LAN & BYOD Configuration

Layer 2 Security

Routing & Internet Access

Overall Network Security

Design Justifications